APPENDIX B

Sensor Specifications

Purpose. This appendix documents the principal sensing elements incorporated into the 3D-PAWS configuration evaluated in this study. The specifications combine manufacturer-stated characteristics for the electronic sensors with the implementation characteristics of the locally fabricated meteorological sensors. Where a sensor is custom fabricated or its final performance depends on calibration and mechanical construction, this is explicitly indicated rather than assigning an unsupported accuracy value.

# B.1 Sensor Suite

| Parameter | Sensor / Device | Measurement principle | Nominal range / resolution | Interface / output | Role in 3D-PAWS | Specification basis |
| --- | --- | --- | --- | --- | --- | --- |
| Air temperature | MCP9808 digital temperature sensor | Digital semiconductor temperature sensing | −40 to +125 °C; selectable resolution down to 0.0625 °C | I²C / SMBus | Primary temperature observation | Manufacturer specification |
| Relative humidity | SHT31 digital humidity/temperature sensor | Capacitive RH sensing with integrated temperature sensing | 0–100 %RH; typical RH accuracy ±2 %RH; temperature accuracy typically ±0.2 °C in the specified range | I²C | Primary humidity observation and auxiliary temperature | Manufacturer specification |
| Atmospheric pressure | BMP280 barometric pressure sensor | Piezo-resistive MEMS pressure sensing | 300–1100 hPa; typical absolute accuracy about ±1 hPa under specified conditions | I²C / SPI | Primary pressure observation | Manufacturer specification |
| Wind speed | 3D-printed cup anemometer with Hall-effect sensing | Cup rotation converted to electrical pulses | System-specific; determined by mechanical geometry and calibration | Pulse / digital input | Wind-speed observation | Local design and field calibration |
| Wind direction | 3D-printed wind vane with AS5600 magnetic angle sensor | Contactless magnetic angular position measurement | 0–360°; 12-bit angular output (4096 positions/revolution) | I²C; analog/PWM also supported by AS5600 | Wind-direction observation | AS5600 manufacturer specification plus local vane calibration |
| Rainfall | 3D-printed tipping-bucket rain gauge | Rainfall collected in a funnel and accumulated through calibrated tipping mechanism | Resolution determined by bucket volume; station-specific calibration | Pulse / digital input | Rainfall occurrence and accumulation | Local fabrication and calibration |
| Solar / optical radiation indicator | SI1145 | Integrated UV-index, ambient-light and infrared sensing | Ambient-light sensing up to approximately 128 klux; integrated UV index | I²C | Solar/optical radiation indicator | Manufacturer specification; application-specific calibration |
| Data acquisition / communications | Particle Boron | Embedded processing with cellular communications | nRF52840 MCU; model-dependent 2G/3G or LTE Cat-M1 cellular connectivity | I²C, SPI, UART, GPIO, cellular | Sensor interfacing, processing and transmission to cloud services | Manufacturer specification |

# B.2 Temperature Sensor – MCP9808

Sensor function. The MCP9808 is used as a digital air-temperature sensing element. The manufacturer specifies a typical accuracy of ±0.25 °C over −40 to +125 °C and a maximum accuracy of ±0.5 °C over −20 to +100 °C, with selectable measurement resolution from 0.5 °C to 0.0625 °C. The sensor communicates through the I²C/SMBus interface and operates from 2.7–5.5 V.

# B.3 Relative Humidity Sensor – SHT31

Sensor function. The SHT31 provides the principal relative-humidity observation and also provides temperature measurements. The SHT31-DIS-B specification gives an operating RH range of 0–100 %RH, typical RH accuracy of ±2 %RH and typical temperature accuracy of ±0.2 °C over the stated temperature range. The device uses I²C communication and is factory calibrated.

# B.4 Atmospheric Pressure Sensor – BMP280

Sensor function. The BMP280 is the principal barometric pressure sensor used for atmospheric pressure observations. Its specified pressure operating range is 300–1100 hPa. Bosch Sensortec specifies typical absolute accuracy of approximately ±1 hPa under 950–1050 hPa and 0–40 °C conditions, with I²C and SPI interfaces.

# B.5 Wind-Speed Sensor

System implementation. Wind speed is measured using a locally fabricated cup anemometer. Rotation of the cups is detected using a Hall-effect sensing arrangement and converted into a wind-speed value by the station firmware. Unlike the integrated electronic sensors, the final wind-speed accuracy is not assigned from a generic component datasheet because it depends on cup geometry, shaft/friction characteristics, pulse counting, installation exposure and calibration. The calibration relationship used by the deployed station should therefore be treated as the authoritative system specification.

# B.6 Wind-Direction Sensor – AS5600

Sensor function. Wind direction is obtained from a locally fabricated wind vane coupled to an AS5600 contactless magnetic rotary position sensor. The AS5600 provides 12-bit angular output, corresponding to 4096 positions over a full 360° range, and supports I²C, analog and PWM outputs. The final wind-direction accuracy of the complete station is dependent on vane alignment, magnet positioning, mechanical coupling and field calibration.

# B.7 Rainfall Sensor – Tipping-Bucket Rain Gauge

System implementation. Rainfall is measured using a locally fabricated tipping-bucket rain gauge. Rainwater collected through the funnel fills a calibrated bucket mechanism; each tip generates a pulse that is converted into rainfall depth. Because the bucket volume and mechanical construction are part of the local design, the rainfall resolution and calibration coefficient are station-specific and should be taken from the relevant calibration record. In the validation analysis, zero rainfall is retained as a valid observation.

# B.8 Solar / Optical Radiation Sensor – SI1145

System implementation. The SI1145 provides optical environmental measurements including ambient light, infrared and UV-index information. The manufacturer specifies an ambient-light dynamic range extending to approximately 128 klux and an integrated UV-index sensor. In the present 3D-PAWS implementation it is treated as a solar/optical radiation indicator rather than as a laboratory-grade pyranometer. Any derived solar-radiation value therefore requires application-specific calibration.

# B.9 Data Acquisition and Communications – Particle Boron

System role. The Particle Boron serves as the station controller and communications device. It interfaces with the meteorological sensors, processes observations and transmits data to the cloud platform. The Boron family is based on the Nordic nRF52840 and provides I²C, SPI, UART and GPIO interfaces. Depending on the specific Boron model deployed, cellular connectivity is provided through 2G/3G or LTE Cat-M1 technology.

# B.10 Sensor Integration and Measurement Chain

The sensing architecture follows a modular design in which individual sensors are connected to the Particle Boron through digital buses, pulse inputs or appropriate signal-conditioning interfaces. The controller timestamps and processes the observations before transmission to the CHORDS platform and the custom monitoring environment. This modular architecture permits individual sensors to be replaced, recalibrated or upgraded without requiring replacement of the complete station.

# B.11 Specification and Calibration Considerations

| Component | Manufacturer specification available | Local calibration required | Validation consideration |
| --- | --- | --- | --- |
| MCP9808 | Yes | Recommended for deployed installation | Temperature comparison and error statistics |
| SHT31 | Yes | Recommended for deployed installation | Humidity comparison and error statistics |
| BMP280 | Yes | Recommended / station dependent | Pressure comparison and derived pressure products |
| Cup anemometer | No single system accuracy | Yes | Wind-speed calibration and paired comparison |
| AS5600 + wind vane | Component specification available | Yes | Direction alignment and angular comparison |
| Tipping bucket | Component-level only | Yes | Bucket calibration and rainfall accumulation comparison |
| SI1145 | Yes | Yes for quantitative radiation conversion | Optical/radiation indicator comparison where available |
| Particle Boron | Yes | No sensor calibration; system configuration required | Data transmission, timestamping and availability |

# B.12 Note on Reported Accuracy

The manufacturer accuracy figures presented in this appendix describe the individual electronic components under specified laboratory or operating conditions. They should not be interpreted as the achieved accuracy of the complete 3D-PAWS station. Field performance is influenced by radiation shielding, sensor placement, installation height, exposure, mechanical construction, calibration, environmental conditions, power stability and communications. The achieved performance is therefore assessed independently through the validation methodology and results presented in Chapters 6 and 7 and Appendix E.

# B.13 Manufacturer and Technical References

Microchip Technology Inc. MCP9808 – 0.5°C Maximum Accuracy Digital Temperature Sensor.

Sensirion AG. SHT3x Digital Humidity and Temperature Sensor Datasheet.

Bosch Sensortec. BMP280 Digital Pressure Sensor Datasheet.

ams OSRAM. AS5600 – 12-bit Programmable Contactless Potentiometer / Magnetic Rotary Position Sensor.

Silicon Labs. Si1145/46/47 – Proximity/UV/Ambient Light Sensor Module with I²C Interface.

Particle. Boron Datasheet and Boron Hardware Documentation.